System Verilog Files

ProgramCounter.sv

Holds the current PC. On every rising clock edge:

- If rst=1 → PC resets to 0x00000000

- Otherwise → PC latches pc_in (whatever the next PC was computed to be)

Starts at 0x00000000 to match your compact RARS memory configuration.

PC_Logic.sv

Decides what the next PC should be based on PCS from the decoder and alu_flags from the ALU:

| PCS | Meaning | PC_src output |
| --- | --- | --- |
| 00 | Normal instruction | PC + 4 |
| 01 | Conditional branch | PC + imm if condition true, else PC + 4 |
| 10 | JAL | PC + imm |
| 11 | JALR | ALU result (rs1 + imm) |

For branches, it checks funct3 to determine the condition (BEQ, BNE, BLT, BGE, BLTU, BGEU) against the 3 ALU flags: {eq, lt, ltu}.

Decoder.sv

The control unit. Reads the instruction and produces all the control signals for the rest of the CPU. Key signals it sets per instruction type:

| Instruction | alu_src_a | alu_src_b | reg_write | mem_write | mem_to_reg | PCS |
| --- | --- | --- | --- | --- | --- | --- |
| R-type | 00 (rs1) | 0 (rs2) | 1 | 0 | 0 | 00 |
| I-type (ADDI etc.) | 00 | 1 (imm) | 1 | 0 | 0 | 00 |
| LW | 00 | 1 | 1 | 0 | 1 | 00 |
| SW | 00 | 1 | 0 | 1 | 0 | 00 |
| Branch | 00 | 0 | 0 | 0 | 0 | 01 |
| JAL | 00 | 1 | 1 | 0 | 0 | 10 |
| JALR | 00 | 1 | 1 | 0 | 0 | 11 |
| LUI | 01 (zero) | 1 | 1 | 0 | 0 | 00 |
| AUIPC | 10 (PC) | 1 | 1 | 0 | 0 | 00 |

Also sets link_reg=1 for JAL/JALR so PC+4 gets written to rd instead of the ALU result.

ALU.sv

Purely combinational. Takes src_a, src_b, and a 4-bit control code, outputs result and 3 flags:

| Control | Operation |
| --- | --- |
| 0000 | ADD |
| 0001 | SLL |
| 0010 | SLT (signed) |
| 0011 | SLTU (unsigned) |
| 0100 | XOR |
| 0101 | SRL |
| 0110 | OR |
| 0111 | AND |
| 1000 | SUB |
| 1001 | MUL |
| 1101 | SRA |

Flags computed independently of the operation: flags[2]=eq, flags[1]=lt (signed), flags[0]=ltu (unsigned).

Extend.sv

Sign-extends the raw instruction bits into a full 32-bit immediate. imm_src from the decoder selects the format:

| imm_src | Type | How it's assembled |
| --- | --- | --- |
| 000 | I-type | {20{sign}, instr[31:20]} |
| 001 | S-type | {20{sign}, instr[31:25], instr[11:7]} |
| 010 | B-type | scrambled bits + appended 0 at bit 0 |
| 011 | U-type | {instr[31:12], 12'b0} (used by LUI and AUIPC) |
| 100 | J-type | scrambled bits + appended 0 at bit 0 |

B and J types append a 0 at bit 0 because branch/jump offsets are always multiples of 2.

RegFile.sv

32 registers, each 32-bit. x0 is hardwired to 0 (read always returns 0, writes to rd=0 are blocked).

- Reads are combinational: RD1 and RD2 update immediately when rs1/rs2 change

- Writes are sequential: on rising clock edge, if we=1 and rd≠0, write WD into reg_bank[rd]

SevenSegDecoder.sv

Drives the 8-digit seven-segment display. Since you can't light all 8 digits at once (only one anode active at a time), it multiplexes rapidly:

- 13-bit counter increments every clock

- When counter wraps to 0, an_to_display increments → moves to next digit

- Each of the 8 digits gets a 4-bit nibble from the 32-bit data input (bits 31:28, 27:24 ... 3:0)

- seg is a combinational lookup table converting that nibble to 7-segment encoding (active low)

- an is the anode select (active low, only 1 bit low at a time)

At 100MHz with a 13-bit counter, each digit is active for 8192 cycles = ~82µs, full refresh rate ~1.5kHz — fast enough to appear solid to the human eye.

RISCV_MMC.sv

The top-level CPU that wires everything together. Key muxes:

- src_a mux: alu_src_a=00 → rs1, 01 → 32'b0 (LUI), 10 → PC (AUIPC)

- src_b mux: alu_src_b=1 → immediate, 0 → rs2

- write_data mux: mem_to_reg=1 → memory data, link_reg=1 → PC+4, else → ALU result

- pc_next mux: 00 → PC+4, 01 → PC+imm, 11 → ALU result & 0xFFFFFFFE

alu_result is exposed as mem_addr in Top_MMC — the ALU computes all memory addresses via ADD.

Top_MMC.sv

The board-level wrapper. Connects the CPU to:

- Instruction ROM (256 words, loaded from AA_IROM.mem) — word-addressed via PC[10:2]

- Data memory (256 words at 0x2000–0x20FF)

- MMIO mapped at fixed addresses (LED, SW, BTN, TIMER, RNG, SS)

- LFSR and timer running continuously every clock

- SevenSegDecoder for the display

- Clock is direct (clk_cpu = clk) for simulation; divide by 16 (counter[3]) for hardware

Assembly Code

INIT

lui  x10, 0x2 / addi x10, x10, 0x400   → x10 = 0x2400 (LED)

addi x11/x12/x13 ...                    → SW, BTN, SS addresses

addi x14, x0, 0                         → score = 0

li   x20, 1                             → RNG seed = 1

Decoder sets alu_src_a=01 (zero) for LUI, alu_src_a=10 (PC) would be AUIPC. Extend produces U-type immediate. ALU adds 0 + imm.

WAIT_START

sw   x0, 0(x10)     → clear LEDs

sw   x0, 0(x13)     → clear SS

lw   x18, 0(x12)    → read BTN from 0x2408

beq  x18, x0, WAIT_START

lw → ALU computes x12 + 0, Top_MMC sees mem_addr=0x2408, returns {27'b0, btnC, btnU, btnL, btnR, btnD} into mem_read_data. CPU writes that into x18 via mem_to_reg=1.

beq → ALU SUBs x18 and x0, sets flags[2]=eq. PC_Logic sees PCS=01, funct3=000, checks flags[2] → branches back if no button pressed.

RELEASE (debounce)

lw   x18, 0(x12)

bne  x18, x0, RELEASE

Waits until button is released. Same hardware path as above but BNE — branches while button still held. Prevents the start press registering as a mole hit.

SET DIFFICULTY

lw   x17, 0(x11)        → read switches (0x2404)

li   x18, 500

mul  x17, x17, x18      → scale switches

li   x18, 5000000

add  x17, x17, x18      → add base timeout

mul uses R-type opcode with funct7=0000001. Decoder sets alu_control=1001 → ALU multiplies. More switches up = bigger x17 = longer countdown = easier game.

NEXT_MOLE (randomness)

add  x20, x20, x16      → mix in last reaction time

li   x18, 13

mul  x20, x20, x18      → scramble

addi x20, x20, 19

andi x15, x20, 0x3      → mole = 0,1,2,3

This is a linear congruential generator. x16 (leftover countdown) feeds into x20 so every round's seed depends on how fast the player reacted — unpredictable. andi masks to bottom 2 bits → picks one of 4 moles.

Light the LED + set expected button

addi x18, x0, 1

sll  x18, x18, x15      → 1 shifted left by mole index

sw   x18, 0(x10)        → write to LED address 0x2400

addi x19, x0, 1

sll  x19, x19, x15      → same shift = expected button bitmask

Top_MMC sees mem_addr=0x2400, mem_we=1 → writes mem_write_data[15:0] into led register. Lights exactly 1 LED matching the mole position. x19 stores which button must be pressed.

WAIT_INPUT (countdown loop)

addi x16, x16, -1       → decrement timer

andi x18, x16, 0xFF

bne  x18, x0, SKIP_SS   → only update display every 256 cycles

slli x18, x14, 16

or   x18, x18, x16

sw   x18, 0(x13)        → score on left digits, countdown on right

SKIP_SS:

beq  x16, x0, GAMEOVER  → timeout

lw   x18, 0(x12)        → read buttons

beq  x18, x0, WAIT_INPUT → no press, keep waiting

bne  x18, x19, GAMEOVER  → wrong button

slli x14, 16 puts score into bits [31:16]. or with x16 puts countdown into bits [15:0].

SevenSegDecoder splits the 32-bit value into 8 nibbles — score shows on left 4 digits, countdown on right 4. Updated only every 256 cycles to avoid thrashing the bus.

CORRECT

addi x14, x14, 1        → score++

slli x18, x14, 16

sw   x18, 0(x13)        → show new score immediately

sw   x0, 0(x10)         → LED off (mole whacked)

li   x1, 3000000

P1: addi x1, x1, -1

bne  x1, x0, P1     → celebration pause ~30ms at 100MHz

li   x18, 100000

sub  x17, x17, x18      → speed up (shorter timeout next round)

li   x18, 1000000

blt  x17, x18, CLAMP    → if timeout < floor, clamp it

jal  x0, NEXT_MOLE

CLAMP: li x17, 1000000

jal x0, NEXT_MOLE

blt uses signed comparison — Decoder sets alu_control=1000 (SUB), PC_Logic checks flags[1] (signed less-than). Prevents timeout going negative.

GAMEOVER

slli x18, x14, 16

sw   x18, 0(x13)        → show final score

li x18, 0xFFFF / li x15, 3

FL_LOOP:

sw x18, 0(x10)      → all 16 LEDs on

[10M cycle delay]

sw x0,  0(x10)      → all LEDs off

[10M cycle delay]

addi x15, x15, -1

bne x15, x0, FL_LOOP → flash 3 times

[150M cycle delay]       → hold score ~1.5s at 100MHz

addi x14, x0, 0         → reset score

jal x0, WAIT_START      → restart

li x18, 0xFFFF is a pseudo-instruction RARS expands to lui + addi. Top_MMC writes mem_write_data[15:0] to led, so all 16 bits go high — all LEDs flash. 150M cycles × 10ns = 1.5 seconds hold time.