NoName Rhythm Game

Game Instructions for Nexys 4 FPGA

# 1. Overview

NoName Rhythm Game is a hardware rhythm game running on a RISC-V CPU implemented on the Nexys 4 FPGA board. Notes scroll across the 8-digit 7-segment display from right to left. Hit each note with the correct pushbutton as it reaches the line marker to score points. Miss too many and it is game over.

# 2. Hardware Layout

| Component | Location | Function |
| --- | --- | --- |
| 16 DIP Switches | Bottom of board | Set random seed (any pattern) |
| 8-digit 7-seg display | Top of board | Note lane + line marker |
| 16 LEDs | Above switches | Lives remaining (16 = full, 0 = game over) |
| 5 Pushbuttons | Centre of board | Start game / hit notes / reset |
| CPU Reset button | Top-left of board | Hard reset — hold to run |

# 3. Start Screen

When the board powers on (or after a reset), the 7-segment display shows:

SEEd XXXX

where XXXX is the current 16-bit value of the DIP switches displayed in hex. This value is used as the random seed for note generation. Different switch patterns produce different note sequences.

- Flip the DIP switches to any pattern you like to choose your seed.

- The display updates live as you move switches.

- btnC (centre button) to lock in the seed and start the game.

- CPU Reset at any time to restart the board from scratch.

# 4. Display Layout

The 8 displays are arranged left to right as AN7 to AN0:

AN7  AN6  AN5  AN4  AN3  AN2  AN1  AN0

[   ] [ - ] [   ] [   ] [   ] [   ] [   ] [   ]

- LINE: always shows a middle dash '-'. This is where you must hit the note.

- note lane: notes spawn at AN0 and travel left one display every 0.5 seconds.

- AN7 (leftmost) is unused during gameplay.

# 5. Note Types

Four types of notes appear on the display. Each note shows a specific segment pattern:

| Nibble | 7-Seg Display | Button to Press |
| --- | --- | --- |
| 2 | Top dash ‘—’ (segment a) | btnU (Up) |
| 3 | Bottom dash ‘_’ (segment d) | btnD (Down) |
| 4 | Left bar ‘¦’ (segments e+f) | btnL (Left) |
| 5 | Right bar ‘¦’ (segments b+c) | btnR (Right) |
| 1 | Middle dash ‘-’ (line marker) | Do NOT press — this is the line |
| 0 | Blank (empty slot) | Do NOT press — no note |

# 6. How to Play

## Hitting a Note

Watch the note travel from right to left. When it reaches AN6 (the line marker), press the matching button:

- btnU

- btnD

- btnL

- btnR

If you press the correct button while the note is at the line, the note disappears and your score increases by 1.

## Missing a Note

If a note passes through the line (AN6) and you do not press the correct button in time, the note moves to AN7 and is counted as a miss. One LED turns off from the right side of the LED bar for each miss.

## Wrong Button

Pressing a button when there is no note at the line has no effect, it is not penalised. Only misses (notes that pass the line unhit) cost lives.

# 7. Lives and Scoring

- 16 lives: all 16 LEDs lit.

- Each miss turns off one LED from the right.

- Your score is the total number of notes successfully hit.

- The note sequence loops after 128 notes, so the game continues indefinitely until all lives are lost.

# 8. Game Over

When all 16 LEDs are off (0 lives remaining):

- The 7-segment display flashes '8888 8888' for approximately 1 second.

- Your final score is then shown in hex on the rightmost digits.

- btnC (centre button) to return to the start screen and play again with a new seed.

# 9. Reset at Any Time

Pressing btnC during gameplay immediately returns to the start screen, but your current score and lives are discarded. This works at any point during the game including during note generation.

# 10. Quick Reference

| Button | Action |
| --- | --- |
| btnC (Centre) | Start game / return to start screen at any time |
| btnU (Up) | Hit top dash note '—' |
| btnD (Down) | Hit bottom dash note '_' |
| btnL (Left) | Hit left bar note '¦' |
| btnR (Right) | Hit right bar note '¦' |
| CPU Reset | Hard reset: hold to run the board |